SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using

simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Dr.V.Parthipan

Annexure A

Debugging &amp; Optimisation Task

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work

and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Debugging &amp; Optimisation Task

- A rapidly growing IT company has shifted its operations to a newly constructed six- storey smart office building. More than 1,200 employees use wireless laptops, smartphones, IP phones, printers, and IoT devices connected through the company’S Wireless Local Area Network (WLAN). The office relies heavily on cloud computing, online meetings, video conferencing, and file sharing, making uninterrupted wireless connectivity essential for daily operations. In addition, the company has a team of field engineers and sales executives who travel across different cities and use 4G/5G Wireless Wide Area Network (WWAN) services to access the company’S servers, customer databases, and business applications from remote locations. During the last few weeks, employees have reported several connectivity issues. Users experience slow internet speeds during peak working hours, frequent Wi-Fi disconnections while moving between floors, and weak wireless signals in conference rooms. Video conferencing applications often freeze or disconnect, affecting communication with international clients. Network administrators have also observed that some wireless access points are overloaded while others remain underutilized, leading to uneven traffic distribution. Furthermore, field engineers working in rural and highway areas frequently lose WWAN connectivity, delaying the upload of maintenance reports and customer information. Security audits reveal that a few wireless devices are still connected using outdated security protocols, exposing sensitive company information to potential cyber threats. The management has requested the network engineering team to investigate these issues, identify the root causes, and optimize both the WLAN and WWAN infrastructure to ensure high performance, secure communication, and seamless connectivity for all users.

ANSWER:

To resolve the connectivity, performance, and security issues in the smart office building, the network engineering team must implement a structured, multi-layered optimization strategy addressing both the Wireless Local Area Network (WLAN) and the Wireless Wide Area Network (WWAN) infrastructure.

Part 1: Root Cause Analysis of Reported Issues

- Slow Internet Speeds During Peak Hours & Overloaded Access Points:

  - Root Cause: Uneven traffic distribution, sticky clients, and lack of automated load balancing across Access Points (APs). Certain APs near heavy-traffic zones are saturated while others handle minimal loads.

- Frequent Wi-Fi Disconnections Between Floors:

  - Root Cause: Poor roaming optimization and lack of proper signal overlap. Clients hold onto an AP from a previous floor ("sticky client syndrome") until the signal degrades entirely, causing session drops.

- Weak Signals in Conference Rooms:

  - Root Cause: Sub-optimal AP placement, signal attenuation caused by walls/glass partitions, or improper channel/power configuration.

- Video Conferencing Freezing/Disconnects:

  - Root Cause: High latency, packet loss, and absence of Quality of Service (QoS) policies prioritizing real-time unified communications traffic over background file downloads.

Scribd

- Field Engineers Losing WWAN Connectivity in Rural/Highway Areas:

  - Root Cause: Dead zones, weak cellular carrier coverage, and lack of carrier redundancy or intelligent multi-path routing on remote devices.

- Security Vulnerabilities (Outdated Protocols):

  - Root Cause: Legacy devices or unmanaged IoT sensors operating on outdated encryption standards (such as WEP or legacy WPA2-PSK) without enterprise-grade security enforcement.

Part 2: WLAN Optimization & Engineering Solutions

1. Access Point (AP) Placement and RF Design

- Site Survey & Density Planning: Conduct a fresh predictive and active site survey across all six stories. Ensure proper AP density to handle 1,200+ concurrent multi-device users.

- Cell Shrinking & Power Tuning: Adjust transmission power levels dynamically to create 15–20% cell overlap between adjacent APs and floors, ensuring smooth roaming without co-channel interference.

2. Roaming and Mobility Enhancements

- Implement 802.11r (Fast BSS Transition): Enable fast roaming protocols so client devices can securely pre-authenticate with target APs before disassociating from current ones, preventing voice/video drops when moving between floors.

- Band Steering & Load Balancing: Enable Band Steering to automatically push capable dual-band clients (laptops, modern smartphones) to the less congested 5 GHz and 6 GHz bands, reserving 2.4 GHz primarily for low-bandwidth IoT devices. Configure aggressive client load balancing on the WLAN controller to evenly distribute users across nearby APs.

3. Quality of Service (QoS) Implementation

- Configure WMM (Wi-Fi Multimedia) and map traffic classes using DSCP (Differentiated Services Code Point) tags. Prioritize real-time audio and video conferencing packets above routine web browsing and file transfers to eliminate jitter and freezing.

4. Network Security Overhaul

- Upgrade to WPA3 Enterprise: Mandate WPA3-Enterprise authentication (utilizing 802.1X with RADIUS/TACACS+) across all corporate laptops and smartphones to eliminate weak pre-shared keys.

- Isolate IoT Devices: Place printers, IP phones, and IoT sensors on segregated Virtual LANs (VLANs) protected by strict Access Control Lists (ACLs) and firewall policies to prevent lateral movement if an IoT device is compromised.

Part 3: WWAN Optimization for Field Engineers

1. Multi-Carrier SIM / Multi-Network Resilience

- Equip field engineers' mobile devices and routers with multi-carrier eSIM technology or dual-SIM setups (e.g., combining major carriers like AT&T, Verizon, or T-Mobile). This allows devices to automatically switch to an active network carrier if another loses signal in rural or highway zones.

2. Intelligent SD-WAN / VPN Failover

- Deploy Software-Defined WAN (SD-WAN) clients or resilient mobile VPN tunnels (such as IKEv2/IPsec with session persistence) on field laptops. This ensures that if a momentary cellular drop occurs, data sessions and active database connections freeze rather than terminate entirely, resuming seamlessly once connectivity is re-established.

3. Compression and Offline Caching

- Enable data compression features on corporate access gateways to reduce bandwidth requirements over poor cellular links, and configure field application clients to support offline data entry with automatic background synchronization when a stable network is detected.

2. A multinational financial organization operates its headquarters, regional offices, and multiple data centers using an Asynchronous Transfer Mode (ATM) network to support mission-critical services such as online banking, video conferencing, Voice over IP (VoIP), and real-time financial transactions. The ATM network uses fixed-size 53-byte cells and establishes Virtual Circuits (VCs) to provide reliable and predictable Quality of Service (QoS). Recently, the organization has experienced several performance issues during peak business hours. Customers report delays in online transactions, voice calls suffer from jitter and occasional packet loss, and video conferencing sessions experience reduced quality. Network monitoring indicates that several ATM switches are operating at high utilization, some Virtual Circuits are congested, and available bandwidth is not being efficiently allocated among different traffic classes. Engineers also discover that certain applications are using inappropriate ATM service categories, leading to increased delay and inefficient resource utilization. The IT department has assigned you as the network engineer to investigate the ATM network, identify the faults, debug the performance issues, and recommend optimization strategies to restore efficient, reliable, and high-quality communication across the enterprise.

ANSWER:

To restore high performance, predictable Quality of Service (QoS), and reliable communication across the financial organization's ATM network, we must analyze the architectural root causes of the reported faults and implement targeted ATM traffic engineering and resource allocation strategies.

Part 1: Root Cause Analysis of Performance Issues

- Delays in Online Transactions:

  - Root Cause: Critical database transactions and real-time financial traffic are likely sharing congested Virtual Circuits (VCs) with high-bandwidth, bursty, best-effort background traffic without proper prioritization or strict traffic shaping.

- VoIP Jitter and Occasional Packet/Cell Loss:

  - Root Cause: Inappropriate selection of ATM service categories (e.g., using ABR or UBR instead of real-time classes) and buffer overflow on heavily utilized ATM switches causing cell delay variation (CDV).

- Reduced Video Conferencing Quality:

  - Root Cause: Inadequate bandwidth allocation or misuse of Variable Bit Rate (VBR) parameters, leading to cell dropping during peak traffic bursts when policing mechanisms trigger.

- High Switch Utilization & Inefficient Bandwidth Distribution:

  - Root Cause: Static Virtual Path (VP) and Virtual Circuit (VC) provisioning that fails to dynamically adapt to traffic demands, coupled with unoptimized policing (Peak Cell Rate [PCR] and Sustainable Cell Rate [SCR] misconfigurations).

- Inappropriate ATM Service Categories:

  - Root Cause: Applications are mapped to non-optimal service classes (e.g., mapping constant-bit-rate or delay-sensitive traffic onto non-real-time or best-effort service categories), resulting in unnecessary cell transfer delay (CTD) and cell loss ratio (CLR) degradation.

Part 2: ATM Traffic Engineering & Optimization Strategies

1. Re-mapping to Correct ATM Service Categories

Align each application with its proper ATM Forum service category to match its specific traffic and QoS profile:

- Voice over IP (VoIP): Map to CBR (Constant Bit Rate) or rt-VBR (Real-Time Variable Bit Rate). This guarantees strict bounds on Cell Transfer Delay (CTD) and Cell Delay Variation (CDV) to eliminate jitter.

- Real-Time Financial Transactions & Video Conferencing: Map to rt-VBR (Real-Time Variable Bit Rate), utilizing parameters like Peak Cell Rate (PCR), Sustainable Cell Rate (SCR), and Maximum Burst Size (MBS) to accommodate bursty traffic while guaranteeing low latency.

- Non-Critical Data / File Transfers: Map to nrt-VBR (Non-Real-Time VBR) or ABR (Available Bit Rate) to utilize remaining bandwidth efficiently using feedback mechanisms.

- General Background Traffic: Map to UBR (Unspecified Bit Rate), ensuring it takes lowest priority and drops first during congestion.

2. Optimizing Traffic Parameters and Policing (PCR, SCR, MBS)

- Reconfigure traffic contracts on core and edge ATM switches. Ensure that PCR and SCR accurately reflect the actual peak and average transmission rates of applications.

- Tune the Maximum Burst Size (MBS) to prevent policing mechanisms (like the Generic Cell Rate Algorithm / Leaky Bucket) from prematurely dropping cells during sudden transaction spikes.

3. Virtual Circuit (VC) and Virtual Path (VP) Re-routing

- Perform load balancing across alternate paths in the ATM core network to offload congested switches.

- Establish dedicated Virtual Paths (VPs) for high-priority traffic aggregates (Voice and Financial Transactions) to simplify routing and reduce processing overhead at intermediate ATM switches.

4. Implementing ATM Traffic Management Controls

- Usage Parameter Control (UPC) / Network Parameter Control (NPC): Enforce strict traffic policing at entry points to prevent rogue applications from exceeding their contracted traffic descriptors and impacting other VCs.

- Priority-Based Queuing and Buffering: Configure multi-tier priority queues on ATM switch output ports, ensuring that real-time service cells (CBR and rt-VBR) are serviced ahead of best-effort (UBR) cells during high utilization periods.

3. A multinational e-commerce company has interconnected its headquarters, regional offices, warehouses, and cloud data centers using a Multiprotocol Label Switching (MPLS) network to ensure fast, secure, and reliable communication. The MPLS infrastructure supports critical business applications, including online order processing, enterprise resource planning (ERP), video conferencing, Voice over IP (VoIP), cloud-based inventory management, and real-time customer support. Recently, the organization has experienced frequent performance issues during peak business hours. Employees report slow access to cloud applications, delayed order processing, poor-quality voice calls, and interruptions during video conferences. Network monitoring reveals that some Label Switched Paths (LSPs) are heavily congested, while other paths remain underutilized. The IT team also discovers that several routers have incorrect label mappings, causing inefficient routing and increased latency. In addition, traffic engineering has not been properly configured, resulting in uneven bandwidth utilization across the MPLS backbone. Since these issues are affecting customer satisfaction and business operations, the network administrator has asked you to investigate the MPLS network, identify the faults, debug the performance problems, and recommend suitable optimization techniques to improve the overall efficiency and reliability of the network.

ANSWER:

To resolve the performance bottlenecks, high latency, and inefficient traffic distribution across the multinational e-commerce company's MPLS network, we must conduct a structured fault analysis and implement advanced MPLS Traffic Engineering (MPLS-TE) and Quality of Service (QoS) strategies.

Part 1: Root Cause Analysis of Performance Issues

- Slow Access to Cloud Applications & Delayed Order Processing:

  - Root Cause: High-priority ERP and transactional traffic are likely traversing congested Label Switched Paths (LSPs) without proper bandwidth reservation or priority queueing, leading to buffer drops and increased round-trip time.

- Poor-Quality Voice Calls & Video Conference Interruptions:

  - Root Cause: Lack of strict Class of Service (CoS) mapping and DiffServ-to-MPLS Experimental (EXP) bit prioritization, causing real-time media streams to suffer from jitter, packet loss, and delay alongside heavy bulk data transfers.

- Congested LSPs vs. Underutilized Paths:

  - Root Cause: Absence of MPLS Traffic Engineering (MPLS-TE). Standard Interior Gateway Protocols (IGPs) like OSPF or IS-IS rely solely on shortest-path metrics (hop count or link bandwidth) rather than actual link utilization, causing traffic to crowd a single path while alternative links sit idle.

- Incorrect Label Mappings & Increased Latency:

  - Root Cause: Misconfigured Label Distribution Protocol (LDP) sessions, stale Forwarding Equivalence Class (FEC) bindings, or routing table desynchronization between Label Switch Routers (LSRs), leading to suboptimal packet forwarding or routing loops.

Part 2: MPLS Network Optimization and Troubleshooting Strategies

1. Troubleshooting and Correcting Label Mappings

- Verify LDP Sessions: Run diagnostic commands (e.g., show mpls ldp neighbor / show mpls forwarding-table) across all LSRs to check for flapping or broken LDP neighbor relationships.

- Clear and Resynchronize LDP Bindings: Reset LDP bindings where mismatches occur to ensure that FEC-to-label mappings are correctly synchronized across the entire MPLS domain, eliminating inefficient routing loops.

2. Implementing MPLS Traffic Engineering (MPLS-TE)

- Enable RSVP-TE: Configure Resource Reservation Protocol-Traffic Engineering (RSVP-TE) on the routing protocol (OSPF-TE or ISIS-TE) to dynamically reserve bandwidth along specific paths.

- Establish Explicit LSPs: Design and configure explicit primary and secondary traffic-engineered paths that bypass congested core links. This shifts heavy traffic (like cloud inventory management and video streams) away from bottlenecks and balances load evenly across underutilized links.

3. Enforcing Quality of Service (QoS) and EXP Bit Marking

- Classify and Mark at the Edge: Inspect and classify traffic at the Customer Edge (CE) / Provider Edge (PE) routers using access control lists (ACLs) and Network-Based Application Recognition (NBAR). Mark real-time traffic (VoIP, Video) with strict Differentiated Services Code Point (DSCP) values.

- Map DSCP to MPLS EXP Bits: Configure the PE routers to map DSCP values directly to the 3-bit MPLS EXP (Experimental) field in the shim header.

- Priority Queueing (PQ) / Low Latency Queueing (LLQ): Implement strict priority queuing in core and distribution routers so that packets carrying high EXP precedence values are serviced first, completely eliminating jitter for voice and video.

4. Bandwidth Protection and Fast Reroute (FRR)

- MPLS Fast Reroute (FRR): Implement link and node protection using MPLS-TE FRR (Detour or Facility backup methods). This ensures that if a core link fails, backup tunnels take over within 50 milliseconds, preventing service interruption for real-time e-commerce transactions and voice sessions.

4. A multinational logistics company operates an automated warehouse where barcode scanners, RFID readers, autonomous robots, and inventory management systems continuously exchange data over a computer network. Every day, thousands of shipment records, customer details, and inventory updates are transmitted between warehouse terminals and the central server. Recently, the company has observed that some shipment records are arriving with incorrect information, barcode scans occasionally fail, and inventory quantities in the database do not always match the physical stock. During peak hours, network monitoring reports an increase in transmission errors caused by electrical interference, signal attenuation, and network congestion. Although some corrupted frames are detected and retransmitted, others remain undetected, resulting in incorrect database entries and delayed order processing. The IT department suspects that the current error detection and correction mechanisms are not configured efficiently. As the network engineer, you are assigned to investigate the transmission errors, identify the weaknesses in the existing error control techniques, and recommend suitable debugging and optimization strategies to improve the reliability and accuracy of data communication.

ANSWER:

To resolve the data corruption issues, inventory mismatches, and transmission errors in the automated warehouse network, we must analyze the physical and data link layer vulnerabilities and implement robust error control and mitigation strategies.

Part 1: Root Cause Analysis of Transmission Errors

- Incorrect Shipment Records & Inventory Mismatches:

  - Root Cause: Undetected bit errors during frame transmission. Weak or insufficient error detection codes (e.g., relying on basic parity bits or weak Checksums instead of robust Cyclic Redundancy Checks) are failing to catch multi-bit corruptions caused by environmental noise.

- Electrical Interference & Signal Attenuation:

  - Root Cause: Unshielded copper cabling routed near heavy warehouse machinery, excessive cable lengths exceeding standards, or poor wireless RF coverage causing high signal-to-noise ratio (SNR) degradation.

- Network Congestion & Failed Barcode Scans:

  - Root Cause: Buffer overflows on switches and access points during peak hours, leading to dropped frames. If automatic repeat request (ARQ) retransmissions are delayed or buffer queues are mismanaged, real-time barcode/RFID data packets time out or fail.

- Undetected Corrupted Frames:

  - Root Cause: Inadequate frame-checking mechanisms or legacy Data Link Layer configurations that lack end-to-end data integrity validation across the application layer.

Part 2: Error Control & Optimization Strategies

1. Upgrading Error Detection Mechanisms

- Implement CRC-32 (Cyclic Redundancy Check): Ensure that Ethernet frames utilize standard 32-bit CRC (IEEE 802.3). CRC-32 detects all single-bit errors, double-bit errors, burst errors of length 32 or less, and 99.99% of larger burst errors, virtually eliminating undetected corrupted frames.

- Application-Layer Checksumming: Implement secondary verification mechanisms (such as MD5, SHA hashes, or cryptographic checksums) within the inventory management application protocol for critical shipment records.

2. Enhancing Error Correction Techniques (ARQ Optimization)

- Selective Repeat ARQ: Upgrade the Data Link / Transport layer flow control to use Selective Repeat ARQ rather than Stop-and-Wait or Go-Back-N. This ensures that only the specific corrupted or lost data frames (like a failed barcode scan packet) are retransmitted without holding up the entire data stream, drastically reducing latency.

3. Physical Layer and Medium Remediation

- Shielded Cabling (STP / S/FTP): Replace unshielded twisted pair (UTP) cables in high-interference warehouse zones with Shielded Twisted Pair (STP) or fiber-optic links to eliminate electrical noise generated by heavy machinery and automated robot motors.

- Signal Attenuation Mitigation: Install fiber optics for long runs between warehouses and the central server to eliminate distance-based signal attenuation. Add industrial-grade Power over Ethernet (PoE) switches or signal repeaters where cable lengths approach limits.

4. Network Congestion and Buffer Management

- Traffic Prioritization (QoS): Configure Strict Priority Queuing at the switch level to give real-time RFID reader and barcode scanner data absolute priority over bulk file transfers or background backups, preventing congestion-related frame drops.

- Buffer Tuning: Increase interface buffer sizes on access switches handling heavy automated terminal traffic to absorb sudden bursts during peak operating hours

5. A leading online education platform conducts live classes and online examinations for more than 15,000 students across different locations. During examinations, the central server simultaneously sends question papers, images, videos, and answer submission acknowledgements to all students over the network. The server is equipped with a high- speed 10 Gbps connection, whereas many students access the system using slower broadband or mobile internet connections. During peak hours, students experience incomplete downloads, delayed page loading, duplicate packets, and repeated retransmissions. Some students receive the next set of data before their devices finish processing the previous one, causing receiver buffer overflow and application crashes. Network administrators also observe an excessive number of retransmissions, increased latency, reduced throughput, and poor bandwidth utilization. The current flow control mechanism is not effectively regulating the transmission rate between the sender and receivers. As the network engineer, you have been assigned to investigate the communication problems, identify the causes of inefficient flow control, debug the existing system, and recommend suitable optimization techniques to ensure reliable and efficient data transmission during high-traffic periods.

ANSWER:

simultaneously sends question papers, images, videos, and answer submission

acknowledgements to all students over the network. The server is equipped with a high-

speed 10 Gbps connection, whereas many students access the system using slower

broadband or mobile internet connections. During peak hours, students experience

incomplete downloads, delayed page loading, duplicate packets, and repeated

retransmissions. Some students receive the next set of data before their devices finish

processing the previous one, causing receiver buffer overflow and application crashes.

Network administrators also observe an excessive number of retransmissions, increased

latency, reduced throughput, and poor bandwidth utilization. The current flow control

mechanism is not effectively regulating the transmission rate between the sender and

receivers. As the network engineer, you have been assigned to investigate the

communication problems, identify the causes of inefficient flow control, debug the existing

system, and recommend suitable optimization techniques to ensure reliable and efficient

data transmission during high-traffic periods.

To resolve the data transmission bottlenecks, buffer overflows, and performance degradation on the online education platform during live examinations, we must analyze the transport and flow control mechanisms and implement robust optimization strategies.

Part 1: Root Cause Analysis of Communication Problems

- Receiver Buffer Overflow & Application Crashes:

  - Root Cause: Speed mismatch (Bottleneck / Fast Sender, Slow Receiver). The high-speed 10 Gbps server is transmitting data faster than students' slower broadband or mobile connections can process, overwhelming their device memory buffers.

- Incomplete Downloads & Delayed Page Loading:

  - Root Cause: Packet loss and congestion collapse caused by inadequate pacing or failure of the sender to dynamically adapt its transmission window size to match varying path capacities.

- Duplicate Packets & Repeated Retransmissions:

  - Root Cause: Aggressive timeout thresholds or premature retransmission timeouts (RTO) combined with poor acknowledgment tracking, leading the server to resend packets that were merely delayed in transit.

- Excessive Retransmissions, Increased Latency, and Reduced Throughput:

  - Root Cause: Inefficient or legacy TCP congestion control algorithms (such as standard TCP Reno) that overreact to minor drops by drastically reducing throughput, coupled with a lack of proper rate-limiting or traffic shaping for individual student sessions.

Part 2: Flow Control and Optimization Strategies

1. Optimizing Transport Layer Flow Control

- Implement Dynamic Sliding Window (TCP Window Scaling): Ensure TCP Window Scaling (RFC 1323) is fully enabled. This allows receivers to advertise a larger window size, but more importantly, enables the sender to dynamically adjust the amount of unacknowledged data in flight based on real-time receiver buffer availability.

- Receive-Side Window Tuning: Configure the client-side applications and operating systems to allocate larger dynamic socket receive buffers so slower devices can comfortably ingest incoming bursts of exam questions and images.

2. Modernizing Congestion Control Algorithms

- Deploy BBR (Bottleneck Bandwidth and RTT) or Cubic: Replace legacy loss-based congestion control algorithms (like Reno) with TCP BBR (developed by Google) or TCP CUBIC. BBR models the network bottleneck bandwidth and round-trip propagation time, allowing the server to transmit at the exact maximum rate the student's connection can handle without filling up buffers and causing packet drops.

3. Traffic Shaping and Rate Limiting at the Server Edge

- Per-Session Rate Shaping: Implement token bucket or leaky bucket traffic shapers on the central server interface. Cap or pace the maximum transmission rate for lower-bandwidth student profiles so data streams continuously and smoothly rather than arriving in uncontrollable multi-gigabit bursts.

- Multicast or CDN Offloading: For static assets (such as examination question paper templates, instructional images, and pre-recorded videos), offload distribution to a Content Delivery Network (CDN) with regional edge servers close to the students, or utilize IP Multicast where applicable to reduce redundant server-to-client streams.

4. Retransmission and Timeout Optimization

- Adaptive Retransmission Timeouts (RTO): Fine-tune Karn's algorithm and smoother RTO estimation parameters on the server to prevent premature timeouts caused by high latency on mobile or rural broadband links.

- Selective Acknowledgments (SACK): Ensure TCP SACK (RFC 2018) is strictly enforced. SACK allows the receiver to inform the server precisely which specific packet segments were received, ensuring the server only retransmits the exact missing pieces rather than an entire data window.