SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Dr. V. Parthipan

Code of Conduct:

I _____________M. V. Preethashri (192524476)_______ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: M. V. Preethashri

Short Answer Test

1.A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

ANSWER:

Given:

- File size = 150 MB = 150×106bytes = 150,000,000 bytes

- Transport Layer segment size = 1500 bytes

- Data Link Layer overhead per frame = 40 bytes

- Bandwidth = 50 Mbps = 50,000,000 bits/s

Step 1: Number of Segments

Number of Segments=150,000,0001500=100,000

Answer: 100,000 segments

Step 2: Number of Frames

Each segment is encapsulated into one frame.

Number of Frames=100,000

Answer: 100,000 frames

Step 3: Total Data Link Layer Overhead

Overhead per frame = 40 bytes

100,000×40=4,000,000 bytes

= 4 MB

Answer: 4,000,000 bytes (4 MB)

Step 4: Total Data Transmitted

Frame size:

1500+40=1540 bytes

Total bytes transmitted:

100,000×1540=154,000,000 bytes

Convert to bits:

154,000,000×8=1,232,000,000 bits

Step 5: Transmission Time

Time=1,232,000,00050,000,000
=24.64 seconds

Estimated Transmission Time = 24.64 seconds

Reliability

- Transport Layer: Segmentation, sequence numbers, ACKs, retransmission, flow control.

- Data Link Layer: Framing, CRC/error detection, local retransmission, node-to-node reliable delivery.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

ANSWER:

Given

- Sliding window size = 6 frames

- Each frame carries = 1024 bytes

- Total frames to transmit = 48 frames

Step 1: Calculate the Number of Transmission Windows

Number of Windows=Total FramesWindow Size
=486=8

Answer: 8 transmission windows

Step 2: Calculate the Total Retransmissions

One frame is lost in every window.

Number of windows = 8

Total Retransmissions=8×1=8

Answer: 8 frames must be retransmitted

Final Answer

- Transmission Windows: 8

- Retransmissions: 8 frames

Flow Control :

- Prevents the sender from overwhelming the receiver.

- Matches the transmission rate with the receiver's capacity.

- Reduces congestion, packet loss, and unnecessary retransmissions.

- Improves overall communication efficiency and reliability

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

ANSWER:

Given

- IP packet size = 12,000 bytes

- MTU = 1500 bytes

- IP header per fragment = 20 bytes

Step 1: Data carried in each fragment

Maximum data per fragment:

1480 bytes

1500 − 20 = 1480 bytes

Step 2: Number of fragments

Original data size = 12,000 bytes

Fragments required:

9 fragments

⌈12,000 / 1480⌉ = 9

First 8 fragments carry 1480 bytes each:

8 × 1480 = 11,840 bytes

Remaining data = 12,000 − 11,840 = 160 bytes

So, the 9th fragment carries 160 bytes of data.

Step 3: Total header overhead

Each fragment gets a new 20-byte IP header.

9 × 20 = 180 bytes

- Fragmentation: The router divides the large packet into smaller fragments that fit the MTU.

- Identification Field: All fragments carry the same packet ID so the destination knows they belong together.

- Fragment Offset: Indicates the position of each fragment in the original packet.

- More Fragments (MF) Flag: Tells whether additional fragments are expected.

- Reassembly: The destination host collects all fragments and reconstructs the original 12,000-byte packet.

- Error Handling: If a fragment is missing, the packet is discarded and higher-layer protocols (such as TCP) can request retransmission

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Given

- Sliding window size = 6 frames

- Data per frame = 1024 bytes

- Total frames transmitted = 48 frames

Step 1: Calculate the Number of Transmission Windows

Number of Windows=Total FramesWindow Size=486=8

Answer: 8 transmission windows

Step 2: Calculate the Total Retransmissions

One frame is lost in every window.

Total Retransmissions=8×1=8

Answer: 8 frames must be retransmitted

Final Answer

| Parameter | Value |
| --- | --- |
| Sliding Window Size | 6 frames |
| Data per Frame | 1024 bytes |
| Total Frames | 48 |
| Number of Transmission Windows | 8 |
| Lost Frames | 1 per window |
| Total Retransmissions | 8 frames |

How Flow Control Improves Communication Efficiency

- Prevents the sender from sending data faster than the receiver can process.

- Avoids buffer overflow at the receiver.

- Reduces packet loss and unnecessary retransmissions.

- Improves network throughput, reliability, and efficient bandwidth utilization.

- Ensures smooth and reliable data transfer between sender and receiver.

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

ANSWER:

Given

- Video file size = 600 MB

- Synchronization checkpoint = every 60 MB

- Failure occurs after 390 MB

Step 1: Calculate the Number of Checkpoints Before Failure

Number of Checkpoints=39060=6

Checkpoints are created at:

60 MB, 120 MB, 180 MB, 240 MB, 300 MB, 360 MB

Answer: 6 checkpoints

Step 2: Calculate the Data to be Retransmitted

Failure occurs at 390 MB.

Last checkpoint = 360 MB

390-360=30 MB

Answer: 30 MB must be retransmitted

Final Answer

| Parameter | Value |
| --- | --- |
| Video File Size | 600 MB |
| Checkpoint Interval | 60 MB |
| Failure Point | 390 MB |
| Checkpoints Created | 6 |
| Data to Retransmit | 30 MB |

Importance of Synchronization in the Session Layer

- Inserts checkpoints during data transmission.

- Allows communication to resume from the last checkpoint after a failure.

- Reduces the amount of data that must be retransmitted.

- Saves time, bandwidth, and network resources.

- Improves the reliability and efficiency of long data transfers.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

ANSWER:

Given

- Original file size = 900 MB

- Compression = 40%

- Encryption overhead = 5% of compressed size

Step 1: Calculate the Compressed File Size

Compression reduces the file by 40%.

Compressed Size=900×(1-0.40)=900×0.60=540 MB

Answer: 540 MB

Step 2: Calculate the Encrypted File Size

Encryption adds 5% overhead.

Encryption Overhead=540×0.05=27 MB
Encrypted File Size=540+27=567 MB

Answer: 567 MB

Step 3: Calculate the Total Percentage Reduction

Reduction in size:

900-567=333 MB

Percentage reduction:

333900×100=37%

Answer: 37% reduction

Final Answer

| Parameter | Value |
| --- | --- |
| Original File Size | 900 MB |
| Compressed File Size | 540 MB |
| Encryption Overhead | 27 MB |
| Encrypted File Size | 567 MB |
| Total Reduction | 333 MB |
| Percentage Reduction | 37% |

How the Presentation Layer Improves Transmission Efficiency and Security:

- Compression reduces the amount of data transmitted, saving bandwidth and reducing transmission time.

- Encryption protects data from unauthorized access during transmission.

- Ensures data confidentiality and integrity.

- Performs data formatting, compression, and encryption to improve both efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

ANSWER:

Given

- Total data = 3 GB = 3000 MB

- Request size = 3 MB

- Throughput = 120 Mbps

Step 1: Calculate the Number of Requests

Number of Requests=30003=1000

Answer: 1000 requests

Step 2: Calculate the Transmission Time

Convert data to bits:

3 GB=3000 MB
3000×8=24,000 Mb

Transmission time:

Time=24,000120=200 seconds

Answer: 200 seconds

Final Answer

| Parameter | Value |
| --- | --- |
| Total Data | 3 GB (3000 MB) |
| Request Size | 3 MB |
| Number of Requests | 1000 |
| Throughput | 120 Mbps |
| Transmission Time | 200 seconds |

How the Application Layer Provides Reliable Services

- Provides services such as FTP, HTTP, SMTP, and DNS to users.

- Ensures reliable file transfer using protocols like FTP.

- Supports authentication and error handling during communication.

- Enables efficient data exchange between applications over the network.

- Provides a user-friendly interface for accessing network services.

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

ANSWER:

Given

- Application Layer delay = 4 ms

- Transport Layer delay = 3 ms

- Network Layer delay = 5 ms

- Data Link Layer delay = 2 ms

- Physical Layer delay = 1 ms

- Propagation delay = 18 ms

- Transmission delay = 12 ms

Step 1: Calculate Total Processing Delay

4+3+5+2+1=15 ms

Processing Delay = 15 ms

Step 2: Calculate Total End-to-End Delay

Total Delay=Processing Delay+Propagation Delay+Transmission Delay
=15+18+12=45 ms

Answer: 45 ms

Final Answer

| Parameter | Value |
| --- | --- |
| Processing Delay | 15 ms |
| Propagation Delay | 18 ms |
| Transmission Delay | 12 ms |
| Total End-to-End Delay | 45 ms |

Contribution of Each OSI Layer

- Application Layer: Provides network services to user applications.

- Transport Layer: Ensures reliable data delivery using segmentation and flow control.

- Network Layer: Performs logical addressing and routing of packets.

- Data Link Layer: Handles framing, error detection, and node-to-node delivery.

- Physical Layer: Transmits data as electrical, optical, or wireless signals over the communication medium.

Final Answer: Total End-to-End Delay = 45 ms.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Given

- Useful data = 80 MB = 80,000,000 bytes

- Frame size = 1600 bytes

- Protocol overhead per frame = 80 bytes

Step 1: Calculate Useful Data per Frame

Useful Data=1600-80=1520 bytes

Step 2: Calculate Number of Frames

Frames=80,000,0001520=52,632 frames

Answer: 52,632 frames

Step 3: Calculate Total Overhead

52,632×80=4,210,560 bytes

Answer: 4,210,560 bytes (≈ 4.21 MB)

Step 4: Calculate Transmission Efficiency

Efficiency=Useful DataTotal Data Transmitted×100

Total data transmitted:

80,000,000+4,210,560=84,210,560 bytes
Efficiency=80,000,00084,210,560×100≈94.99%

Answer: ≈ 95%

Final Answer

| Parameter | Value |
| --- | --- |
| Useful Data | 80 MB |
| Frame Size | 1600 bytes |
| Overhead per Frame | 80 bytes |
| Useful Data per Frame | 1520 bytes |
| Number of Frames | 52,632 |
| Total Overhead | 4,210,560 bytes (≈ 4.21 MB) |
| Transmission Efficiency | ≈ 95% |

Effect of Protocol Overhead on Network Performance

- Protocol overhead reduces the amount of useful data carried in each frame.

- Higher overhead increases bandwidth usage.

- It reduces transmission efficiency and can increase transmission time.

- However, overhead is necessary for addressing, error detection, routing, and reliable communication across the network

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

ANSWER:

Given

- Original file size = 240 MB = 240,000,000 bytes

- Compression = 30%

- Segment size = 1400 bytes

- Data Link Layer overhead = 60 bytes/frame

Step 1: Calculate the Compressed File Size

Compressed Size=240×(1-0.30)=240×0.70=168 MB
168 MB=168,000,000 bytes

Answer: 168 MB (168,000,000 bytes)

Step 2: Calculate the Number of Segments

Segments=168,000,0001400=120,000

Answer: 120,000 segments

Step 3: Calculate the Total Protocol Overhead

Overhead=120,000×60=7,200,000 bytes

Answer: 7,200,000 bytes (7.2 MB)

Step 4: Calculate the Final Data Transmitted

168,000,000+7,200,000=175,200,000 bytes
=175.2 MB

Answer: 175.2 MB (175,200,000 bytes)

Final Answer

| Parameter | Value |
| --- | --- |
| Original File Size | 240 MB |
| Compressed File Size | 168 MB |
| Segment Size | 1400 bytes |
| Number of Segments | 120,000 |
| Overhead per Frame | 60 bytes |
| Total Protocol Overhead | 7,200,000 bytes (7.2 MB) |
| Final Data Transmitted | 175.2 MB (175,200,000 bytes) |

How Multiple OSI Layers Work Together

- Presentation Layer: Compresses and encrypts data for efficient and secure transmission.

- Transport Layer: Divides data into segments and ensures reliable delivery using acknowledgments and flow control.

- Network Layer: Routes packets from source to destination.

- Data Link Layer: Adds frames, performs error detection, and enables reliable node-to-node communication.

- Physical Layer: Transmits the final data as electrical, optical, or wireless signals.

Final Result:

- Compressed File Size = 168 MB

- Number of Segments = 120,000

- Total Protocol Overhead = 7.2 MB

- Final Data Transmitted = 175.2 MB