SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

Annexure A

Constraint-Based Problem Solving

Code of Conduct:

I _M.VPREETHASHRI(192524476)____ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:  M.VPREETHASHRI

Annexure A

Constraint-Based Problem Solving

- A  multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

ANSWER:

1. Remote Rural Branch Office – Internet Connectivity and Security

Problem Analysis

The multinational company is establishing a branch office in a remote rural area where high-speed wired Internet is not available. The network must support:

- Cloud-based applications

- Video conferencing

- Secure communication with headquarters

- Continuous business operations

- Limited connectivity budget

- Future expansion

Recommended Internet Connection

A 4G/5G wireless broadband connection should be used as the primary Internet connection where cellular coverage is available. A satellite Internet connection can be used as a backup if cellular coverage is unreliable.

A dual-WAN setup can automatically switch between the primary and backup connections when the primary connection fails.

Recommended Network Devices

- 4G/5G Router

  - Provides Internet connectivity through the cellular network.

  - Supports multiple users and VPN connections.

- Backup Satellite/Secondary WAN Router

  - Provides connectivity during cellular outages.

  - Improves business continuity.

- Gigabit Ethernet Switch

  - Connects computers, printers, IP phones, servers, and access points.

  - VLAN support can be used for network segmentation.

- Wireless Access Points

  - Provide Wi-Fi connectivity to employees.

  - Use modern Wi-Fi standards for better performance.

- Enterprise Firewall

  - Controls incoming and outgoing traffic.

  - Blocks unauthorized access.

  - Supports VPN and security policies.

- VPN Gateway

  - Creates an encrypted connection between the branch and headquarters.

  - Protects sensitive company data over the public Internet.

Security Mechanisms

- Site-to-site IPsec VPN between branch and headquarters

- Firewall with intrusion prevention

- WPA3-Enterprise for Wi-Fi security

- Multi-factor authentication (MFA) for important services

- VLAN-based network segmentation

- Regular firmware and security updates

- Strong password and access-control policies

- Network monitoring and logging

Constraint-Based Justification

| Constraint | Solution |
| --- | --- |
| Limited budget | 4G/5G broadband is generally cheaper to deploy than wired infrastructure |
| Remote location | Wireless/satellite connectivity avoids dependence on wired infrastructure |
| Bandwidth limitation | QoS prioritizes video conferencing and business-critical applications |
| Reliability | Dual-WAN with automatic failover provides backup connectivity |
| Security | Firewall, VPN, VLANs, WPA3, and MFA protect business information |
| Future scalability | Additional access points, switches, users, and higher-bandwidth connections can be added |

QoS Configuration

Since bandwidth may be limited, Quality of Service (QoS) should prioritize:

- Voice and video conferencing

- VPN traffic to headquarters

- Cloud business applications

- Normal web browsing

- Large downloads and non-business traffic

Conclusion

The best solution is a 4G/5G-based primary connection with a backup satellite or secondary wireless connection, combined with a firewall, managed switch, Wi-Fi access points, VPN, VLANs, WPA3, and MFA. This design provides a good balance between cost, reliability, security, bandwidth, and future scalability.

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

ANSWER:

University Campus Wireless Network – 3,000+ Students

Problem Analysis

The university needs campus-wide Wi-Fi for more than 3,000 students across:

- Classrooms

- Laboratories

- Libraries

- Hostels

The major constraints are:

- Large number of simultaneous users

- Limited number of access points

- Signal interference

- Secure authentication

- Good coverage

- Limited budget

Therefore, simply increasing the number of access points is not possible. The available APs must be used efficiently.

Recommended Wireless Architecture

Use a centrally managed enterprise Wi-Fi network with strategically positioned wireless access points.

The APs should be connected to managed PoE switches and controlled through a wireless LAN controller or cloud-based management system.

Access Point Placement

Access points should be placed based on a wireless site survey rather than simply placing them at equal distances.

Recommended locations:

- Classrooms: ceiling-mounted APs positioned near the center

- Laboratories: APs positioned according to user density and equipment interference

- Libraries: additional capacity around high-user areas

- Hostels: APs distributed floor-by-floor

- Corridors: avoid relying on corridors as the main coverage source

High-density areas should receive priority because many students may connect simultaneously.

Frequency Band Selection

2.4 GHz

Advantages:

- Longer range

- Better wall penetration

Disadvantages:

- More interference

- Fewer non-overlapping channels

- Lower capacity

Therefore, 2.4 GHz should mainly be used for devices that require longer range.

5 GHz

Advantages:

- More available channels

- Higher data rates

- Less interference than 2.4 GHz

Therefore, 5 GHz should be the primary band for student devices.

6 GHz

If the university's devices and Wi-Fi infrastructure support Wi-Fi 6E/7, the 6 GHz band can be used in high-density areas. It provides additional spectrum and helps reduce congestion.

Interference Reduction

To minimize interference:

- Perform a wireless site survey.

- Use appropriate channel planning.

- Reduce excessive AP transmit power.

- Avoid overlapping neighboring channels.

- Prefer 5 GHz/6 GHz for high-density users.

- Use automatic radio-frequency management where appropriate.

- Avoid placing APs near electrical equipment and thick obstacles.

Authentication and Security

The university should use WPA3-Enterprise with 802.1X authentication.

A centralized RADIUS server can authenticate students using their university credentials.

For example:

Student → Access Point → RADIUS Server → Authentication → Network Access

Different VLANs can be created for:

- Students

- Faculty

- Administration

- Guests

- IoT devices

Efficient Network Design

The university should implement:

- Wi-Fi 6/6E or newer enterprise APs

- Managed PoE switches

- Central wireless controller/cloud management

- WPA3-Enterprise

- 802.1X/RADIUS authentication

- VLAN segmentation

- QoS

- Band steering

- Client load balancing

- Captive portal for guests

Constraint-Based Justification

| Constraint | Solution |
| --- | --- |
| 3,000+ users | High-capacity Wi-Fi APs and proper client distribution |
| Limited APs | Strategic AP placement based on site survey |
| Interference | Channel planning and transmit-power management |
| High traffic | Prefer 5 GHz/6 GHz and use QoS |
| Security | WPA3-Enterprise + 802.1X + RADIUS |
| Budget | Deploy APs according to user density instead of blanket coverage |
| Scalability | Central controller and managed switches allow future AP expansion |

Conclusion

An efficient university WLAN should use strategically positioned Wi-Fi 6/6E access points, with 5 GHz/6 GHz as the primary bands, proper channel planning, and WPA3-Enterprise with 802.1X/RADIUS authentication. This provides a balance between coverage, performance, security, interference reduction, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

ANSWER:

Financial Institution – Secure Network Architecture

Problem Analysis

A financial institution handles highly sensitive customer and banking information. Therefore, the network must provide strong security while maintaining good performance and controlling operational costs.

The main requirements are:

- Protection of customer information

- Secure online banking services

- Employee access control

- Network segmentation

- Intrusion detection

- Strong authentication

- Regulatory compliance

- High performance

- Controlled operational cost

Recommended Secure Network Architecture

A defense-in-depth architecture should be implemented.

A simplified architecture is:

Internet

↓

Edge Router

↓

Perimeter Firewall

↓

DMZ

↓

Internal Firewall

↓

Core Network

↓

Separate VLANs / Security Zones

- Employee Network

- Customer Data Network

- Banking Application Network

- Server Network

- Management Network

- Guest Network

1. Network Segmentation

Network segmentation is one of the most important security mechanisms.

Use VLANs and firewall-based security zones to separate different types of systems.

For example:

| Network | Purpose |
| --- | --- |
| Employee VLAN | Normal employee systems |
| Customer Data VLAN | Sensitive customer information |
| Banking Application VLAN | Banking applications |
| Server VLAN | Internal servers |
| Management VLAN | Network administration |
| Guest VLAN | Guest Internet access |

Sensitive systems should not communicate directly with ordinary employee or guest networks.

2. DMZ

Public-facing services such as:

- Online banking web servers

- Public web servers

- External application gateways

should be placed in a DMZ (Demilitarized Zone).

The DMZ separates Internet-facing systems from the internal network.

This prevents an attacker who compromises a public-facing server from immediately reaching internal customer databases.

3. Firewall Deployment

Use next-generation firewalls (NGFWs) at important network boundaries.

The firewall should provide:

- Stateful packet inspection

- Application-level filtering

- Access-control rules

- VPN support

- Intrusion prevention

- Malware filtering

- Logging and monitoring

A firewall should follow the principle of least privilege, allowing only required traffic.

4. Authentication

Strong authentication should be used for employees and administrators.

Recommended mechanisms:

- Multi-factor authentication (MFA)

- Strong passwords

- 802.1X for network access

- Role-based access control

- Privileged access management for administrators

For example:

Username + Password + OTP/Security Key

This reduces the risk of unauthorized access even if a password is stolen.

5. Intrusion Detection and Prevention

Deploy IDS/IPS to monitor network traffic.

IDS — Intrusion Detection System

- Detects suspicious activity

- Generates alerts

- Helps security teams investigate attacks

IPS — Intrusion Prevention System

- Detects suspicious activity

- Can automatically block malicious traffic

The institution should use centralized security monitoring such as a SIEM to collect and analyze logs from firewalls, servers, authentication systems, and network devices.

6. Encryption

Sensitive information should be encrypted:

- TLS for web and application communication

- IPsec VPN for secure remote/site-to-site connections

- Encryption of sensitive data at rest

- Secure administrative protocols such as SSH

7. Performance Considerations

Security should not create unnecessary network delays.

Therefore:

- Use high-performance firewalls.

- Apply firewall rules efficiently.

- Avoid unnecessary traffic inspection.

- Use load balancing for critical applications.

- Prioritize important banking applications using QoS.

- Monitor bandwidth and device CPU utilization.

8. Compliance

Because financial institutions handle sensitive information, the organization should maintain:

- Access-control policies

- Audit logs

- Security monitoring

- Data encryption

- Regular vulnerability assessments

- Security updates and patch management

- Periodic security audits

- Incident-response procedures

The exact regulatory requirements depend on the institution's country and applicable financial/data-protection regulations.

Constraint-Based Justification

| Constraint | Solution |
| --- | --- |
| High security requirement | Defense-in-depth architecture |
| Sensitive customer data | VLANs, firewalls, encryption and access control |
| Online banking | DMZ and application security |
| Unauthorized access | MFA, 802.1X and RBAC |
| Network attacks | IDS/IPS |
| Performance | High-performance firewalls and optimized rules |
| Compliance | Logging, auditing, encryption and monitoring |
| Limited budget | Use centralized security management and prioritize critical systems |

Conclusion

The financial institution should implement a defense-in-depth network architecture using VLAN segmentation, DMZ, next-generation firewalls, MFA, 802.1X, IDS/IPS, encryption, RBAC, and centralized monitoring. This architecture protects sensitive financial data while maintaining network performance and controlling operational costs.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |