SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE CODE: CSA0713                     COURSE NAME: COMPUTER   NETWORKS

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

Code of Conduct:

I _M. V. Preethashri (192524476)    (Name / Reg No) certify that this submission is my original work

and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: M.V.Preethashri

Annexure A

ANALYTICAL PROBLEM SOLVING

1. Suppose we want to transmit the message 11100011 and protect it from errors using the

CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver’s CRC calculation? How does the receiver know that an error has occurred?

ANSWER:

i.) Determine the Transmitted Message Using CRC

1. Setup the Polynomials

- Message polynomial (Mx): Represented by the bit string 11100011, which corresponds to:

Mx=x7+x6+x5+x1+1

- Generator polynomial (Gx): Given as X3+1, which corresponds to the bit string 1001.

- Degree of the generator (r): The highest power of Xin Gxis 3, so we append r=3zeros (000) to the original message.

2. Prepare the Dividend

- Extended message: 11100011000

3. Perform Polynomial Long Division (Modulo-2 Arithmetic)

We divide the extended message by the generator 1001 using XOR operations (no carries or borrows):

Plaintext

1 1 0 1 0 1 1 0 0

---------------------------

1 0 0 1 | 1 1 1 0 0 0 1 1 0 0 0

- 1 0 0 1

---------

0 1 0 1 0

- 0 0 0 0

---------

1 0 1 0 0

- 1 0 0 1

---------

0 0 1 1 1

- 0 0 0 0

---------

0 1 1 1 1

- 0 0 0 0

---------

1 1 1 0

- 1 0 0 1

---------

0 1 1 0 0

- 1 0 0 1

---------

1 0 1 0

- 1 0 0 1

---------

0 0 1 1  <-- Remainder (CRC Checksum)

4. Determine the Transmitted Message

- Remainder (Rx): 011

- Final Transmitted Message: Combine the original message with the remainder:

Transmitted Data=Original Message+Remainder=11100011011

ii.) Error Detection at the Receiver

1. Simulate the Error

- Received Message (with leftmost bit inverted): The original transmitted message is 11100011011. Inverting the leftmost bit changes the first 1 to a 0, resulting in:

Received Data=01100011011

2. Receiver's CRC Calculation

The receiver takes the received bit string and divides it by the same generator polynomial Gx=1001using modulo-2 long division:

Plaintext

0 1 1 1 0 1 1 0 0

---------------------------

1 0 0 1 | 0 1 1 0 0 0 1 1 0 1 1

- 0 0 0 0

---------

1 1 0 0

- 1 0 0 1

---------

1 0 1 0

- 1 0 0 1

---------

0 1 1 0

- 0 0 0 0

---------

1 0 0 1

- 1 0 0 1

---------

0 0 1 0

- 0 0 0 0

---------

0 1 0 1

- 0 0 0 0

---------

1 0 1 1

- 1 0 0 1

---------

0 1 0  <-- Result of Receiver's CRC Calculation

- Result of the Receiver's Calculation: The resulting remainder is 010 (which is non-zero).

3. How the Receiver Knows an Error Occurred

- Non-Zero Remainder: In CRC error detection, a valid transmission must yield a remainder of all zeros (000) when the received codeword is divided by the generator polynomial.

- Detection: Because the receiver calculated a non-zero remainder (010 instead of 000), it successfully detects that the data has been corrupted during transmission and rejects the packet or requests retransmission.

2. a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

ANSWER:

a) Calculating Network Throughput

To find the throughput, we need to calculate the average number of bits successfully transmitted per second.

- Given data:

  - Average number of frames per minute = 12,000 frames/min

  - Average bits per frame = 10,000 bits/frame

  - Bandwidth = 10 Mbps (Note: Bandwidth is the maximum capacity, whereas throughput is the actual measured performance).

- Calculate total bits per minute:

Bits per minute=12,000 frames/min×10,000 bits/frame=120,000,000 bits/min

- Convert bits per minute to bits per second (bps):

Throughput=120,000,000 bits60 seconds=2,000,000 bps=2 Mbps

Answer: The throughput of this network is 2 Mbps.

b) Finding the Channel Capacity

To find the maximum theoretical channel capacity of a noisy channel, we use Shannon's Capacity Theorem.

- Given data:

  - Bandwidth (B) = 3000 Hz (calculated as 3300-300)

  - Signal-to-noise ratio (SNR) = 3162

- Apply Shannon's formula:

C=Blog21+SNR

- Substitute the values:

C=3000×log21+3162

C=3000×log23163

- Calculate the logarithm:

log23163≈11.623

- Calculate final capacity:

C=3000×11.623≈34,869 bps or approximately 34.87 kbps

Answer: The channel capacity is approximately 34,869 bps (or 34.87 kbps)

- A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients’ heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

ANSWER:

Comprehensive Analysis of Hamming (12,8) Code in Wireless Patient Monitoring

In a modern multispecialty hospital, real-time wireless patient-monitoring systems are critical infrastructure. They continuously stream vital signs—such as heart rate and blood pressure—to a central nursing station or intensive care unit (ICU). Because these devices operate over wireless channels, they are inherently susceptible to electromagnetic interference (EMI) caused by other medical equipment, mobile devices, or architectural barriers, which can flip bits during transmission.

To maintain high data integrity without the latency introduced by retransmission requests (Automatic Repeat reQuest or ARQ), the hospital utilizes the Hamming (12,8) error-correcting code.

1. Structural Breakdown of Hamming (12,8)

The notation Hamming n,k defines a linear block code where:

- n=12: The total number of bits in the transmitted code word (packet length).

- k=8: The number of actual payload data bits containing the patient's vital sign information.

- r=n-k=4: The number of redundant parity (or check) bits added to the data.

Determining Parity Bit Requirements

For any Hamming code, the number of check bits (r) must satisfy the relation:

2r≥n+1

Substituting r=4:

24=16≥12+1=13

Since 16≥13, four parity bits are mathematically sufficient to uniquely identify any single-bit error across a 12-bit block.

- Code Rate (R):

R=kn=812≈0.67

This indicates that 67% of the transmitted bandwidth is dedicated to actual patient data, while 33% is used for error control overhead.

2. Mechanics of Encoding and Transmission

When a patient-monitoring device measures a vital sign, it processes an 8-bit data block (D1,D2,D3,D4,D5,D6,D7,D8).

- Parity Bit Placement: The 4 parity bits (P1,P2,P4,P8) are inserted at positions that are powers of 2 (positions 1, 2, 4, and 8) within the 12-bit frame. The remaining positions are occupied by the data bits.

- Even/Odd Parity Calculation: Each parity bit is calculated to cover a specific subset of data bits based on binary position overlapping.

- Wireless Transmission: The completed 12-bit codeword is modulated and broadcasted over the wireless medium to the central nursing station.

3. Error Detection and Correction (Syndrome Decoding)

When electromagnetic interference flips a single bit during transit, the receiving station performs syndrome decoding to fix the issue instantly:

- Syndrome Calculation: The receiver recalculates the parity bits from the incoming 12-bit word and compares them against the received parity bits. This comparison yields a 4-bit binary value called the syndrome.

- Zero Syndrome (0000): Indicates that no errors occurred; the packet is accepted immediately.

- Non-Zero Syndrome: The binary value of the syndrome directly points to the exact bit position (1 through 12) that was corrupted by interference.

- Instant Correction: The receiver flips the erroneous bit back to its original state (complementing it) before forwarding the clean data to the doctor's monitor.

4. Real-World Justification in Healthcare

- Eliminating Latency: In emergency medical scenarios, waiting for a packet to be corrupted, detected, dropped, and retransmitted (ARQ) can introduce dangerous delays. Hamming (12,8) allows the receiver to correct errors on-the-fly, ensuring zero-delay continuous monitoring.

- Patient Safety: A corrupted blood pressure reading (e.g., flipping a bit that changes a normal reading into a hypertensive crisis or vice versa) could cause false alarms or missed interventions. Error-correcting codes protect against transient noise, ensuring diagnostic accuracy.

- Limitations: While Hamming (12,8) is highly efficient for single-bit errors, it cannot correct multi-bit errors if electromagnetic interference causes a continuous "burst" of errors within the same frame. For environments with severe, sustained interference, more advanced block codes or convolutional codes are typically paired with interleaving

4. a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the

given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

ANSWER:

a) IPv4 Subnetting Calculations

- Given Network: 172.16.10.0/24

- New Subnet Mask: /27 (which is 255.255.255.224)

- Total Number of Subnets:

  - Original prefix length = 24

  - New prefix length = 27

  - Bits borrowed for subnetting (s) = 27-24=3 bits

  - Total subnets = 2s=23=8 subnets

- Number of Usable Hosts Per Subnet:

  - Total bits in an IPv4 address = 32

  - Host bits remaining (h) = 32-27=5 bits

  - Total IP addresses per subnet = 2h=25=32

  - Usable hosts per subnet = 2h-2=32-2=30 usable hosts

- Subnet for IP Address 172.16.10.78:

  - Block size (subnet increment) = 256-224=32

  - The subnets range as follows:

    - Subnet 0: 172.16.10.0 to 172.16.10.31

    - Subnet 1: 172.16.10.32 to 172.16.10.63

    - Subnet 2: 172.16.10.64 to 172.16.10.95

    - Subnet 3: 172.16.10.96 to 172.16.10.127 (and so on...)

  - Since 78 falls between 64 and 95, the IP address belongs to the 172.16.10.64 subnet.

- Network and Broadcast Address for that Subnet:

  - Network Address: 172.16.10.64

  - Broadcast Address: 172.16.10.95 (the last address in the block)

- Does 172.16.10.95 fall within the same subnet range?

  - Yes. As determined above, 172.16.10.95 is the broadcast address of this specific subnet (172.16.10.64/27), meaning it technically resides within the boundary of this subnet range.

b) IPv6 Addressing and Calculations

- Given IPv6 Address: 2001:0db8:0000:0000:0000:ff00:0042:8329

- Compression to Shortest Form:

  - Apply rule 1: Omit leading zeros in each 16-bit block: 2001:db8:0:0:0:ff00:42:8329

  - Apply rule 2: Replace the longest contiguous sequence of all-zero blocks with a double colon (::): 2001:db8::ff00:42:8329

  - Shortest form: 2001:db8::ff00:42:8329

- Expansion Back to Full Notation:

  - Restore leading zeros to ensure every group has 4 hexadecimal digits and expand the :: back to explicit zero blocks:

  - Full notation: 2001:0db8:0000:0000:0000:ff00:0042:8329

- Network Prefix with a /64 Prefix Length:

  - A /64 prefix means the first 64 bits (the first 4 hextets) are reserved for the network identifier.

  - Network Prefix: 2001:db8::/64 (or fully expanded: 2001:0db8:0000:0000::/64)

- Total Number of Possible Host Addresses:

  - Total bits in IPv6 = 128

  - Host bits remaining = 128-64=64 bits

  - Total available host addresses = 264 (approximately 1.84×1019 or 18.4 quintillion addresses).

5. A university is conducting an online semester examination for 8,000 students

simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high- speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver

buffer overflow, unnecessary retransmissions, increased network traffic, and delays in

delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender’s transmission speed and the receiver’s processing capability affects communication.

ANSWER:

Comprehensive Technical Analysis of the University Examination Network Failure

The scenario described—where an 8,000-student online examination experiences widespread freezing, incomplete downloads, and network collapse—highlights a classic systems-engineering failure rooted in Flow Control Breakdown, Buffer Overflow, and Asymmetric Network Capabilities.

Below is an in-depth analysis of the primary networking problems at play and how the severe mismatch between the sender’s transmission speed and the receivers’ processing capabilities causes communication to break down.

1. Identification of the Primary Networking Problems

While multiple symptoms manifest simultaneously, the root networking failures can be categorized into two primary phenomena:

- Flow Control Failure (Receiver Overwhelm): The server treats all 8,000 clients uniformly, pushing data at rates capacity-matched to a high-speed data center rather than adapting to the constrained consumption rates of individual student devices.

- Buffer Bloat and Packet Loss: Due to the inability of slower links (such as 5–10 Mbps mobile networks) to drain incoming data fast enough, intermediate buffers and local operating system receive buffers overflow, forcing packets to be dropped.

2. Deep Dive: How the Speed Mismatch Affects Communication

The core of the issue lies in the monumental disparity between a 10 Gbps server pipe and client links operating at 5 to 100 Mbps. This creates a catastrophic cascading failure across the OSI stack:

A. The Collapse of the Receiver Buffer

- The Mechanism: When the server begins transmitting encrypted question papers simultaneously, it opens TCP streams to all 8,000 students. TCP relies on a sliding window mechanism, but initially, or under aggressive burst configurations, the sender may overwhelm the receiver's allocated RAM buffer.

- The Impact: A student on a 5 Mbps mobile network cannot pull data from the network interface card (NIC) into the application layer (decryption and rendering) fast enough. The local TCP receive buffer fills to 100% capacity. Once full, any newly arriving segments from the server must be dropped by the client's operating system because there is simply nowhere to store them.

B. The Retransmission Storm and Traffic Amplification

- The Mechanism: When packets are dropped due to buffer overflow, the receiving device does not send back an acknowledgment (ACK) for those segments. The server, interpreting the missing ACK as network packet loss, triggers its timeout mechanisms and retransmits the data.

- The Impact: This creates a vicious feedback loop. The server pours more data onto an already saturated 5–10 Mbps link—sending both new packets and duplicate retransmissions. The link becomes completely choked with redundant traffic, driving the effective throughput (goodput) down to near zero.

C. CPU Starvation and Application Freezing

- The Mechanism: Decrypting and rendering large question papers simultaneously while handling a high-frequency barrage of corrupted or overlapping TCP packets demands significant CPU resources on the student's device (especially on mobile phones or older laptops).

- The Impact: As the network stack becomes bogged down handling interrupts for thousands of incoming, mismanaged packets, the operating system starves the browser or examination portal application of CPU cycles. This manifests on the user end as a "frozen" interface, delayed click responses, and a failure to render the exam questions.

D. TCP Flow Control (Window Size) Mismatch

- The Mechanism: TCP uses a "Window Size" field in the packet header to advertise how much buffer space the receiver has available. If working correctly, a slow receiver advertises a small window (or a Zero Window Size), forcing the sender to pause.

- The Impact: In high-concurrency, sudden-burst scenarios, if the server ignores or fails to poll window updates quickly enough, or if the client's system is too heavily locked up to update its window size dynamically, the server continues its high-speed transmission regardless, bypassing the intended safety valve of flow control.

3. Architectural Deficiencies in the Scenario

- Lack of Traffic Shaping and Throttling: The university server treated a heterogeneous audience homogeneously. Pushing data at maximum capacity without checking client-side absorption rates ignores the reality of public internet diversity.

- Absence of Staged Delivery or CDNs: Broadcasting heavy encrypted payloads to 8,000 concurrent users from a single origin server overburdens the server's egress capacity and fails to leverage edge caching or Content Delivery Networks (CDNs) that can throttle and deliver payloads progressively.

4. Conclusion and Remediation

The examination network collapse proves that bandwidth is not throughput. Even though the university possessed a robust 10 Gbps backbone, the actual performance experienced by the students was throttled by the slowest links and the lack of proper flow-control enforcement.

To prevent this, systems must implement adaptive bitrate streaming, staged login windows (staggering the exam start times by a few minutes for different batches of students), and strict TCP window scaling to ensure the sender never transmits faster than the weakest receiver can process.